Horringer Play Park – Inspection report follow up 2025-10-01

The meeting is about an operational inspection of the play facilities at Horringer, detailing safety findings and maintenance recommendations.

Site Inspection Overview and Findings

The inspection conducted on September 4, 2025, revealed various findings across multiple items at the site, with a focus on safety and maintenance.

General Site Condition and Risks

- The site is partially overlooked by local properties. ​

- Risk assessment rated at 10 - Low Risk. ​

- Some accessible features exist, but challenges remain for individuals with disabilities. ​

Findings Related to Grass Mats and Surface Conditions

- Grass mats are silted and compacted, potentially affecting impact absorption; HIC testing recommended. ​

- Trip hazards identified at surface edges; reinstatement of surrounding levels needed. ​

- Tree debris present on surfacing; removal and maintenance required. ​

- A tree stump remains after removal; stump removal is necessary. ​

Signage Issues and Recommendations

- A sign needs refixing and should be relocated near the entrance gate. ​

- The PSPO sign is incorrectly sited; it should be installed on or near the gate. ​

- Projecting bolt threads present; excess thread length should be removed and deburred. ​

Gate and Fence Condition

- The self-closing gate is satisfactory with no work required.

- The combination gate has eroded surfaces exposing foundations; surrounding surfaces need reinstatement. ​

- A padlock is missing on the maintenance gate; a padlock should be provided. ​

- The bow top fence is damaged; monitoring for further deterioration is advised. ​

Bench and Picnic Table Maintenance

- Benches show signs of erosion around the area, which may become slippery; reinstatement is needed. ​

- Evidence of rot in the timber of benches; monitoring for further deterioration is required. ​

- Picnic tables have tree debris on the surface; removal and maintenance are necessary.

- Erosion in some areas of picnic tables may lead to slippery conditions; surface reinstatement is needed. ​

Equipment Condition and Maintenance Needs

- The climbing frame has multiple findings, including timber splits and surface corrosion; monitoring and treatment are recommended.

- The crawling tunnel has tree debris and algae; both need removal and cleaning.

- The car equipment shows algae and minor surface damage; cleaning and monitoring are required. ​

- The spring hippo is satisfactory with no work required.

Multi Use Games Area and Goal End Findings

- The area around the goal end has eroded, which may become slippery; reinstatement is necessary. ​

- The tree canopy overhangs the equipment, requiring a lift to ensure a minimum clearance of 2.0m. ​

- Movement in the foundations of the goal end should be monitored for deterioration. ​

- Evidence of rot in the timber of the goal end requires monitoring and potential replacement. ​

Next Actions

BY to give area clean and resolve single issue recommendations.

BY to check all trip hazards and signs of damage for suggested repair.

Check condition of single large metal fence pole which has become bent, Might need professional tosort out

Suggestions.

Buy some padlocks and a big brush (BY to source)

Consider what to do with 5 a side goal currently chained to fence and there is only one goal with a bedraggled net